# ALTServer

Dual Pentiumä Processor-based Server

with PCI and EISA I/O

ALTServer

C1F100A2M16

ALTServer

A Full-featured Server -

An Entry-Level Price

Intel’s ALTServer was designed from the beginning to provide superior server performance at an entry-level price. The feature rich design delivers the multi-processing power to run the most demanding applications and easily handles the data throughput required by nearly all server functions. The ALTServer also integrates server management capabilities and security features, while supporting multiple network environments.

Symmetric Multi-Processing Power

Tap into the power of either one or two Pentium™ processors at iCOMPä index 825/100 MHz. Users get maximum performance from having second level write-back cache modules, which are available in asynchronous and burst modes up to 512 KB.

Single processor versions of the ALTServer are easy to upgrade by dropping a second processor into the Zero Insertion Force (ZIF) socket.

Server Architecture

The architecture of the ALTServer has been designed for superior data handling and maximum throughput. The high performance I/O subsystem allows transfers of up to 132 MB/second across the PCI local bus and the EISA bus. An integrated Fast/Wide SCSI-2 controller moves data quickly to and from your peripherals. And peripherals can even exchange data without CPU intervention, thanks to a special interrupt controller.

| Features | Benefits |
| --- | --- |
| · Single or Dual Pentium Processors at iCOMPä index 825/100 MHz | · Processing power for the most demanding of server tasks. |
| · Integrated I/O, graphics, and wide SCSI | · Feature rich design at entry level price |
| · PCI local bus | · Tap into the power of the CPU with high performance PCI add-in cards |
| · Server management features | · Local server management allows user control and security |
| · Six internal and four external bays | · Peripheral expansion capability |
| · Network Operating System certifications | - Proven server performance for Windows NT, OS/2, Novell, Solaris and SCO UNIX |

Expansion for Server Needs

Eight expansion slots are available for RAID disk controllers, networking and other add-in cards. Two of these slots are dedicated connections to the high-performance PCI local bus and five slots are on the EISA bus. The eighth slot accepts either a PCI or EISA card.

The ALTServer offers four external and six internal bays to allow easy integration of hard drives, tape drives, CD-ROMs, and other devices.

Cooling and power requirements are met by two fans and a 230 watt power supply. Or you can order optional I/O connector cards to use the Fast/Wide SCSI-2 interface with external devices.

Integrated Features and Flexibility

The ALTServer baseboard integrates all I/O functions: two serial ports and one  ECP-capable parallel port, plus floppy, IDE, keyboard and mouse controllers - plus on-board video - leaving all slots open for add-in cards.

The	board also incorporates on-board server management features such as voltage and temperature monitoring, error logging, Shared Memory Interface (SMI) interrupt steering, and memory remapping. A software package will be available to help implement all of these features.

A World-class Server

Intel’s ALTServer offers quick time-to-market anywhere in the world. The product has been certified by the major product safety organizations and regulatory agencies in North America, Europe and Japan. The Flash BIOS upgrade and EISA Configuration Utilities even support five languages.

And, no matter where you install the ALTServer, you can be assured that it’s been fully tested and certified as compatible with Windows NT, OS/2, Novell NetWare, UNIXWare, SCO UNIX and the Solaris operating systems.

Server Security

A variety of software and hardware features protect your network data and the server itself from unauthorized access. Included are keyboard and idle console password protection, video blanking, floppy write protection, chassis locks and power/reset switch disabling.

BOARD FEATURES

ALTServer Specifications

PROCESSOR / CACHE

Processors	Single or Dual Pentiumä processors

at iCOMP index 825/100 MHz

Performance Upgrade	Pentium OverDriveä Processors

Future Pentium Processors

Cache	256 or 512 KB, synchronous or 			asynchronous direct-mapped,

write-back L2 cache

SYSTEM  MEMORY

Standard  Memory	Eight 72-pin sockets for tin-lead DRAM 		SIMMs (up to 256 MB total)

SIMM Types	4 MB, 8 MB, 16 MB, 32 MB

(Must fill RAM in Pairs)

Access Time	70 ns or faster

Architecture	64 bit interleaved

EXPANSION SLOTS

Description	Five dedicated EISA  Bus Master slots

Two dedicated PCI  Bus Master slots

One slot shared EISA/PCI slot

SYSTEM BIOS

BIOS Type 	AMIBIOS in Flash memory

Special Features	IDE auto-configuration, PCI auto-			configuration and BIOS password

security.  Supports five languages.

INTEGRATED I/O

I/O Controller	Intel AIP processor

Two Serial Ports	Async, RS232-C, 9-pin, 16C550

compatible

Parallel Port	One multi-mode parallel port with 25-pin 		DSUB connector offering Centronics and 		Extended Capabilities Port support

Keyboard/Mouse	PS/2-style keyboard and mouse interface

Floppy Controller	Integrated floppy drive controller with 		2.88 MB support

Disk Controller	Integrated IDE hard drive controller

Graphics Controller	Integrated PCI / Cirrus 5430 Graphics 			controller with 512 KB on board, 				expandable to 1024 KB VRAM to support 			1024X768 X1M color mode

SCSI Host Adapter	Adaptec 7870  PCI / Fast Wide SCSI-2 			Controller supports 20MB/sec transfer 			rates, stacks 16 SCSI commands

Interrupt Controller	Intel INCA Chip.  Directs 32 interrupts

SYSTEM

Height	19.3” (49.0 cm)

Width	8.3” (21 cm)

Depth	17.7” (45.0 cm)

Weight	33lbs (15 kg), without peripherals

Drive Bays

External	3 @ 5.25"; 1/2 height (1.6")

1 @ 3.5"; 1.0” height

Internal	6 @ 3.5"; 1.0" height

Cooling	Two Fans (Power Supply, Chassis front)

ELECTRICAL

AC Voltage/Frequency	115v/60Hz; 230v/50Hz (switchable)

DC Power Supply	230w

+5V	30.0A maximum continuous

+12V	8.0A maximum continuous, 12.0A Peak

-5V	0.5A maximum continuous

-12V	0.5A maximum continuous

ENVIRONMENT

Ambient Temperature

Operating	+10°C to +35°C

Non-Operating	-40°C to +70°C ambient

Relative Humidity

Operating	85% RH @ 35°C

Non-Operating	95% @ 35°C

Altitude

Operating	to 10,000 feet (3,048 m)

Non-Operating	to 50,000 feet (15,240 m)

Acoustics	45 dB (peripherals idle, at 1 meter)

Static Discharge	25 kv with no component damage

REGULATIONS

Meets or exceeds the following requirements:

Safety

U.S.	UL 1950, 1st edition

Canada	CSA 950

Europe	TUV to EN 60950 with ZHI/618

Ergonomics IEC950 with IEC 380

EMI/RFI

U.S.	FCC CFR 47 Part 15, Level B

Canada	DOC CRC c, 1374 Class B

Europe	VDE 0871 Class B, CISPR-22, Class B

Japan	VCCI, CISPR-22, Class B

ORDERING INFORMATION

C1F100A2M16	ALTServer with one 100 MHz Pentium

processor, 256KB asynchronous cache,

16 MB RAM, Floppy

WEXTRISERBLK	Wide SCSI External I/O Connector Card (10 pack)

NEXTRISERBLK	Narrow SCSI External Style I/O Card (10 pack)

For more information or the number of your nearest Intel sales office, call 800-628-8686 in the U.S. and Canada.

Intel Corporation disclaims all warranties and liabilities for the use of this document and the information contained herein, and assumes no responsibility for any errors that may appear in this document. Intel makes no commitment to update the information contained here, and may make changes at any time without notice. There are no express or implied licenses granted hereunder to any intellectual property rights of Intel Corporation or others to design or fabricate Intel integrated circuits or integrated circuits based on the information in this document.

*other brands and trademarks are the property of their respective owners.

ãINTEL CORPORATION 1994		PRELIMINARY - SUBJECT TO CHANGE

FaxBack document 4405 (Rev 0.2, 12/12/94)