# Classic/PCI LP

Low Profile Desktop using

i486ä, IntelSX2ä, or IntelDX2ä Processors

High Performance and Integration

486LP4S25

486LP4S250

486LP8D266

µ §The Classic/PCI Low Profile offers tremendous flexibility to OEMs seeking high performance at a low system cost . Up to five add-in cards can be used, including two PCI local bus cards to meet the customer's need for high performance devices. Three bays are available for drives or peripherals.

An integrated PCI IDE controller and a Cirrus Logic CL-GD5430 graphics chip  ensure blazing disk transfers and video performance.

A highly integrated chip set and up to 256 KB of write back cache maximize the performance of the processor - from the i486äSX/25 through the IntelDX2ä processor at iCOMPä  index 297 \ 66 MHz. The system is upgradable to the Pentiumä OverDriveä processor, as well as a future OverDrive processor also based on Pentium technology.

| Features | Benefits |
| --- | --- |
| · Support for i486 and IntelDX2 processors | · Multiple price-performance points using fast i486 technology chips |
| · Cirrus 5430 PCI Graphics | · High performance for graphics-intensive applications |
| · Integrated PCI IDE | · Offers performance comparable to SCSI |
| · Two PCI slots on riser card | · Tap into the power of the CPU with high performance PCI add-in cards |
| · ISA Plug and Play, PCI and IDE autoconfiguration | · Simplified integration and configuration |
| · EPA Energy Star* compliance | · Money-saving and energy-efficient design |
| · Ultra-fast system boot time | · Reduced waiting time on start-up |

PCI Local Bus - Performance and Compatibility for the Future

Classic/PCI LP

PCI local bus provides industry-leading performance and compatibility. The 32-bit, 33 MHz pathway to the CPU offers performance unmatched by other local bus architectures.Backed by over 200 leading PC, add-in card, and chip manufacturers, PCI is clearly defined to ensure complete compatibility. The PCI local bus also provides a roadmap to the future. With provisions for 64-bit data paths and 3.3 volt CPUs, you can feel comfortable basing your solution on the PCI local bus.

Autoconfiguration Cuts

Support Costs

Adding expansion cards, memory and hard drives is as simple as plugging them in. Support for ISA Plug and Play and full PCI autoconfiguration helps dramatically reduce end user problems and support costs. The system also provides automatic IDE and memory sizing and configuration.

Energy Star* Compliance Saves Money and the Environment.

Because it supports the Advanced Power Management (APM) specification, the Classic/PCI Low Profile enables any "APM-aware" operating system to control the power consumption. Users can put the system into a low-power sleep mode to save money and energy, and also can use the integrated Desktop Power Management Standard (DPMS) graphics subsystem to put an Energy Star compliant monitor into sleep mode.

Features That Power Users Need

Along with the PCI slots, PCI IDE and second level write back cache, the Classic/PCI ED also supports fast modems (above 9.6 Kb/s) using 16550 compatible serial ports. It also offers the features of an Extended Capability Port (ECP) for compatibility with the Microsoft at Work* standard.

Power users also will appreciate the system's support for 2.88 MB floppy drives.

Intel's High Standards

We understand that OEMs often stretch the limits of PC performance by using a wide variety of peripherals, operating systems, and applications, so our products are thoroughly tested in a wide variety of environments.

Furthermore, policies are in place to ensure product stability. For example, we can provide up to 60 days advance notice of engineering changes.

Outstanding Service and Support

IntelTechDirect provides factory direct support to help you get to market faster. The service includes monthly product updates, FaxBack* service, a technical hotline staffed with trained engineers, and a 24-hour electronic bulletin board.

Every Intel system comes with a 12-month warranty and has several options available for extended coverage and service.

PROCESSOR/CACHE

Classic/PCI LP Specifications

Processors	IntelDX2 processor at

iCOMP index 297 \ 66 MHz

IntelSX2 processor-180 \ 50 MHz

i486 SX processor-100 \ 25 MHz

Upgrades 	Intel Pentium OverDrive Processor

Cache	0, 128, or 256K write-back cache

SYSTEM MEMORY

Standard Memory	Two sockets for tin-lead DRAM SIMMs

Burst mode read/write support, 32-bit 		memory path, fast page DRAM  only.

Memory Size	1 MB min. to 64 MB max.

SIMM Type	1 Mb, 2 Mb, 4 Mb, 8 Mb

x32 (default) or x36. 70ns or faster.

Architecture	32-bit data path. Mix/match SIMM sizes.

INTEGRATED GRAPHICS

Graphics Controller	PCI-based Cirrus 5430 (SVGA)

Video Memory	1 MB expandable to 2 MB

Memory Type	70ns video DRAM.

Compatible With	8514/A, VGA, EGA, HGC, MDA

VGA Connector	15-pin board edge connector

INTEGRATED I/O

I/O Controller	Intel 82091AA (AIP) Super I/O controller

Two Serial Ports	Async, RS-232C, 9-pin, NS16550

compatible with 16-Byte FIFO's

Parallel Port	Centronics compatible 25-pin			bi-directional or output only

Enhanced Parallel Port (EPP)

Extended Capability Port (ECP)

Hard Disk Controller	PCI local bus IDE

Floppy Controller	1.44, 1.2, and 2.88 MB, 765A compatible

Keyboard Controller	Phoenix 8042PE

Keyboard Port	PS/2 compatible

Mouse Port	PS/2 compatible

Real Time Clock	Dallas DS12887, 128 bytes CMOS RAM

EXPANSION SLOTS

Description	One riser slot in standard LPX location 		with two PCI slots and three ISA slots.

SYSTEM BIOS

BIOS Type	Flash AMIBIOS

Hard Disk Driver	IDE, auto-configuring

Special Features	ISA Plug and Play

APM and DPMS power management

JUMPERS	Flash recovery, Flash write protect,

password clear, CMOS clear,

CPU 3.3V, color/mono,	CMOS setup

enable, cache size, CPU clock speed,

Pentium OverDrive cache enable,

CPU type

SYSTEM

Height	4.3" (11.7 cm)

Width	17.2" (43.7 cm)

Depth	16.2" (41.1 cm)

Weight	20 lbs (9.1 kg) without peripherals

Drive Bays	2 @ 5.25"; 1/2 height (1.6") external

1 @ 3.5"; 1/3 height (1.0") external

1 @ 3.5"; 1/2 height (1.6") internal

Floppy Disk Drive (standard)

Footprint	3.5"; 1/3 height (1.0")

Capacity	1.44 MB high density

ELECTRICAL

AC Voltage/Frequency	115v/60Hz; 230v/50Hz (switchable)

DC Power Supply	145w

+5V	18.0A maximum continuous

+12V	4.2A maximum continuous

-12V	0.3A maximum continuous

-5V	0.3A maximum continuous

ENVIRONMENT

Ambient Temperature

Operating	+10°C to +35°C ambient

Non-Operating	-40°C to +70°C ambient

Relative Humidity

Operating	85% @ 35°C

Non-Operating	95% @ 35°C

Altitude

Operating	to 10,000 feet (3,048 m)

Non-Operating	to 50,000 feet (15,240 m)

Acoustics	40 dB (peripherals idle, at 1 meter)

Static Discharge	20 kv with no component damage

REGULATIONS

Meets or exceeds the following requirements:

Safety	U.S.	UL 1950, 1st edition

Canada	CSA 950

Europe	TUV to EN 60950, IEC 950

EMI/RFI	U.S.	FCC CFR 47 Part 15, Level B

Canada	DOC CRC c. 1374 Class B

Europe	VDE 0871 Class B, CISPR-22 Class B

Japan	VCCI, CISPR-22 Class B

ORDERING INFORMATION

486LP8D266	System with IntelDX2 processor-297 \ 66,

8 MB RAM, 1.44MB floppy, no cache.

486LP4S250	System with IntelSX2 processor-180 \ 50,

4 MB RAM, 1.44MB floppy, no cache.

486LP4S25	System with i486 SX processor-100 \ 25,

4 MB RAM, 1.44MB floppy, no cache.

For more information or the number of your nearest Intel sales office, call 800-628-8686 (good in the U.S. and Canada).

Intel Corporation disclaims all warranties and liabilities for the use of this document and the information contained herein, and assumes no responsibility for any errors that may appear in this document. Intel makes no commitment to update the information contained here, and may make changes at any time without notice. There are no express or implied licenses granted hereunder to any intellectual property rights of Intel Corporation or others to design or fabricate Intel integrated circuits or integrated circuits based on the information in this document. i486, i487, OverDrive, Pentium, and iCOMP are trademarks of Intel.

*other brands and trademarks are the property of their respective owners.

ãINTEL CORPORATION 1994		FaxBack document 4305 (rev 1.1, 5/30/94)