# PD440FX

Pentium® II processor-based ATX Board

Time to Market with Pentium II processor performance

The PD440FX motherboard offers OEMs and system integrators the opportunity to deliver “Time to Market Solutions” featuring the performance of the Pentium II processor.  The Pentium II processor delivers the best of Intel’s next-generation processors -- the power of the Pentium Pro processor plus the capabilities of MMX™ Technology for better, smoother, and more realistic media experiences.

The motherboard was designed with flexibility in mind to meet the needs for high performance platforms.  The PD440FX features optional on-board Yamaha OPL3-SA3 audio and dual Universal Serial Bus (USB) connectors enabling  the board for today’s multimedia applications and connectivity to tomorrow’s.  The PD440FX is based on the ATX form factor, which provides easy OEM integration, convenient access for service and upgrades, and superior thermal and emission performance.

Enhanced Manageability: Lowering the Total Cost of Ownership

With the corporate desktop in mind, the PD440FX motherboard was designed with mangeability in mind.  The PD440FX integrates an on-board hardware management ASIC, enhanced Desktop Management Interface (DMI) BIOs, and LANDesk® DMI instrumentation that help reduce support costs through manageability and serviceability.   Through secure remote monitoring, these features, coupled with management software such as LANDesk Client Manager, allow the network administrator to remotely access the user’s system and perform fault prediction, recovery, software installation and configuration tasks.  This reduces the need for service calls which aids in lowering  the total cost of ownership (TCO) in the business environment. Error Checking and Correction (ECC) memory to guard against data corruption.

| Features |  | Benefits |
| --- | --- | --- |
| · Pentium® II processor | · The power of the Pentium Pro plus the capabilities of MMX technology |  |
| · Hardware Management ASIC | · Remote monitoring of system conditions for lower total cost of ownership |  |
| · Universal Serial Bus (USB) connectors | · Expand and simplify PC connectivity while enabling new peripherals |  |
| · 4 72-pin SIMM† sockets | · Flexibility for support up to 256 MB of extended data out (EDO) memory |  |
| · ATX form factor | · Easy integration and upgradeability, reduces total system costs |  |
| · 3 PCI, 2 ISA, and 1 shared slot | · Allows ample room for custom configuration of peripherals |  |
| · Error Checking & Correction (ECC) memory | · Guards against data corruption |  |

PROCESSOR/CACHE

Processor Supported	Pentium II ® processor in S.E.C. cartridge

with 512 KB or 256KB L2 cache

233 Mhz, 266 Mhz, and future  				upgrades

SYSTEM MEMORY

Memory Capacity	Four 72-pin sockets for tin-lead DRAM SIMMs

2 banks (SIMMs filled in pairs for 64 bit bus)

Memory Size	Supports up to 256 MB of memory

8 MB minimum (two 4 MB SIMMs)

256 MB maximum (four 64 MB SIMMs)

SIMM Sizes	4 MB, 8 MB, 16 MB, 32 MB, 64 MB

Access Speed	50ns or 60ns Extended Data Out (EDO) ,			ECC support with ECC and Parity memory

Memory Voltage	5 volts

INTEGRATED AUDIO*	Yamaha OPL3-SA3 (YMF715)

Plug and Play with 16-bit addressing

INTEGRATED PCI/ISA IDE Xccelerator

Dual Channel	2 independent channels, 4 IDE devices,

Fast-IDE Interface	Mode 0, PIO Mode 3 and Mode 4 IDE

Hard Drive and CD-ROM support

USB*	2 standard stacked USB connectors

INTEGRATED SUPER I/O

Controller	National 87307 ISA Plug and Play

Two Serial Ports	Async, RS-232C, PC16C550A compatible

Parallel Port	Compatible / ECP / EPP

IEEE 1284 standard

Floppy Controller	1.44 MB, 2.88 MB, 3-Mode support

Keyboard/Mouse Cntrler	8042A compatible

Real Time Clock	Century Reg. ± 13 minutes/year accuracy

APM/APC	Power On, Off, Sleep, Resume features

Infra Red Interface	IrDA compatible and consumer IR compliant

SYSTEM BIOS

BIOS Type	2 Mb Flash EEPROM with Intel BIOS

Special Features	Plug and Play; IDE drive autoconfigure;

APM 1.2, DMI 2.0, ECC/Parity support;

Multilingual support

PCI CHIP SET 	Intel 82440FX PCIset

Intel 82441FX PMC

Intel 82442FX DBX

Intel 82371SB PIIX3

JUMPERS AND FRONT PANEL CONNECTORS

Connectors	Speaker, Reset, Power LEDs, HD LED,

infrared, power on/off, sleep/resume

Jumpers	Password clear, CMOS clear, setup access,

BIOS recovery, CPU/Bus frequency

HARDWARE MONITOR*	On-board sensors for remote monitoring of

PS voltages, Fan speed, motherboard 				temperature and chassis intrusion

EXPANSION SLOTS (all full length)

Description	Three dedicated PCI slots

Two dedicated ISA slots

One shared slot for either ISA or PCI

MECHANICAL

Board Style	ATX standard mounting holes

Integrated I/O connectors include:

two serial ports, one parallel port, PS/2

style keyboard and mouse connectors,

audio stack and two USB connectors

Board Size	9.6” x 12.0” (24.4 cm x 30.5 cm)

ATX information available at http://www.teleport.com/~atx

BASEBOARD POWER REQUIREMENTS

+3.3V	Tolerance ±5%, typical current 0.6A

+5V	Tolerance ±5%, typical current 7.0A

+12V	Tolerance ±5%, typical current 0.6A

-5V	Tolerance ±10%, typical current 0.01A

-12V	Tolerance ±10%, typical current 0.1A

ENVIRONMENT

Operating Temperature	0°C to +55°C

Storage Temperature	-40°C to +70°C

REGULATIONS

Safety regulations:

U.S & Canada	UL 1950 - CSA 950-95,

U.S. & Canadian recognition component marks

Europe	UL Classified to IEC 950

EMI/RFI regulations:

Europe	CE Mark

This product complies with European Directive 89/336/EEC via standards EN 55 022 and EN 50 082-1 when tested in a representative chassis.

ORDERING INFORMATION

For configurations available in your area, please contact your Intel field sales representative.

Reference Intel’s web page: http://www.intel.com

Information in this document is provided in connection with Intel products. No license, express or implied, by estoppel or otherwise, to any intellectual property rights is granted by this document. Except as provided in Intel’s Terms and Conditions of Sale for such products, Intel assumes no liability whatsoever, and Intel disclaims any express or implied warranty, relating to sale and/or use of Intel products including liability or warranties relating to fitness for a particular purpose, merchantability, or infringement of any patent, copyright or other intellectual property right. Intel products are not intended for use in medical, life saving, or life sustaining applications. Intel retains the right to make changes to specifications and product descriptions at any time, without notice. This product may contain design defects or errors known as errata. Current characterized errata are available on request.

*Third-party brands and names are the property of their respective owners.

ãINTEL CORPORATION 1997		ORDER NUMBER 282981-001