Xpress Modular Platforms

# System Baseboard & CPU Module Quick Reference

(FaxBACK document 4190)

System Baseboards:

|  | Premium/MX | Mainstream/LX | Current Deskside | Current Desktop | Rackmount |
| --- | --- | --- | --- | --- | --- |
| Slots | 8 | 8 | 8 | 6 | 8/6 |
| Watts | 384 watts | 230 watts | 384 watts | 215 watts | 300 or 396 watts |
| Internal Bays | 8 | 5 | 8 | 1 | 0 |
| External Bays | 3 | 3 | 3 | 3 | 4 |
| LCD Support | Yes | No | Yes | No | Yes(8)/No(6) |
| On Board SCSI | Yes | Optional | No | No | Yes(8)/No(6) August '93 |
| SMP Support | Yes | No | No | No | No |
| Fast EISA | Yes* | Yes* | No | No | Yes* |
| All EISA Masters | Yes | Yes | No | No | Yes |
| Installed Memory | 8, 16mb | 8,16mb | 8,16mb | 4,16mb | 4,8mb |
| SCSI Distribution | Yes | Optional | Optional | No | No |
| XDiagnostic SW | Yes | Optional | Optional | Optional | Optional |
| Product Codes | XBASE8TE8F-C XBASE8TE16F-C | XLXBASE8TE8F** XLXBASE8TE16F | XBASE8TE8F-B XBASE8TE16F-B | XBASE6E4F-B XBASE6E16F-B | RXBASE6E4P1 RXBASE8E8P2 |

*Requires MECA III ASIC on Baseboard.

**Apply an "S" at the end of the Product Code for the SCSI Option.

CPU  Modules:

|  | i486SX/25 | i486SX/33 | i486DX/33 | i486DX2/66 | i486DX/50 |
| --- | --- | --- | --- | --- | --- |
| Cache | Upgrade | Yes | Upgrade | Yes | Yes |
| Cache Size* | 128KB(only) | 128KB(only) | 128KB(only) | 128KB(only) | 256KB(only) |
| Cache Type | Write-Through | Write-Through | Write-Through | Write-Through | Write-Back |
| Primary CPU | Integrated 196 pin PQFP | Integrated 196 pin PQFP | Integrated 196 pin PQFP | 238 pin PGA Site | 168 pin PGA |
| OverDrive Ready | Yes | Yes | Yes | Yes | No |
| OverDrive Site | 238 pin ZIF | 238 pin ZIF | 238 pin ZIF | 238 pin ZIF | No |
| Product Code | BXCPU 486SX25-B | BXCPU 486SX33-B | BXCPU 48633-B | BXCPU 486DX266-B | BXCPU 48650-B |

*Refer to Faxback Document 4143 (User-Installable Upgrades) for recommended SRAM vendors.

TITLE:		Xpress System Baseboards & CPU Modules Quick Reference

INTEL REF:	[4190]SYSFAXBACK #:	4190PRODUCT:	Xpress Modular PlatformsDATE/VER:	7/2/93 Ver 1.0RELATED:	Check FaxBACK (800)628-2283KEYWORDS:	System Baseboards, CPU Modules