Highest Performance

BXCPU2XPENT66The BXCPU2XPENT66 extends the Xpress family of modular platforms into the multi-processing market. The highest performance module in the Xpress portfolio features two 66 MHz Pentium microprocessors operating in either symmetric or asymmetric mode, offering over 180 MIPs of processing power  for the most demanding transaction processing applications.

Key features

·	Two 66 MHz Pentium microprocessors

·	Two 256 KB write-back L2 caches

·	Cache-to-cache transfer (data snarfing)

·	Symmetric mode support (-C baseboard)

·	Support for major operating systems

BXCPUPENT60 Specifications

CPU

Family	Pentium processor class

Frequency MHz	66 MHz

L1 Cache	8 KB data, write-back

8 KB code

Voltage	5.4 V (self-regulated)

OverDriveä Processor Socket

Electrically compatible, 2 x LIF

Cache Controller

Intel 82489DX APICs	PC + MP

Cache Controller

Component	Intel 82496

Mode	Write-back

Two-way set associative

Cache

Size	2 x 256KB

Component	Intel 82491

Parity	None

Voltage	5.0 V

Interface

Xpress 32-bit

Mechanical

Width	12.3"

Height	4.5"

µ §Performance

SPECint92	TBD

SPECfp92	TBD

MIPs	180

OS Support

Symmetric*	SCO MPX,Windows NT,

NOvell UNIXWare MP,

IBM OS/2 2.1, Solaris

*requires -C baseboard or later

Asymmetric	MS Windows NT

(HAL modification required)

Regulatory

FCC Class B

For more information call 1 800 INTEL4U

(Good in the US and Canada).

Intel Corporation assumes no responsibility for the use of any circuitry, other than circuitry embodied in an Intel product.  Intel486 and Pentium are trademarks of Intel.  Other brands and names are the property of their respective owners. Intel disclaims all warranties and liabilities for the use of this document  and the information contained herein, and assumes no responsibility for any errors which may appear in this document.

FaxBack document 4295 (Rev 1.0, 3/7/94)