High Performance

BXCPUPENT60The BXCPUPENT60 is a high performance Xpress CPU module featuring a 60 MHz Pentium processor and is designed for transaction processing applications.

The 60 MHz Pentium processor module offers a new price/performance point in the modular and scalable Xpress line of servers, desktops and rackmountable systems. The second-level cache is among the top performers available today.

Key features

·	60 MHz Internal/External CPU speed

·	256 KB write-back L2 cache

·	Zero wait-state two-way set associative L2 cache

·	Backward compatibility with the installed base of Xpress platforms

BXCPUPENT60 Specifications

CPU

Family	Pentium processor class

Frequency MHz	60 MHz

L1 Cache	8 KB data, write-back

8 KB code

Bus data/address	64/32 bit

Voltage	5.0 V

Cache Controller

Component	Intel 82495

Mode	Write-back

Two-way set associative

Zero wait state

Cache

Size	256KB

Component	Intel 82491

Parity	None

Quantity	Eight (8)

Voltage	5.0 V

Interface

Xpress 32-bit

TITLE	XpressPentium processor Module

FAXBACK#	4292DATE/VER	2/4/94, Ver. 1.0

Mechanical

Width	7.325"

Height	4.85"

µ §Performance

SPECint92	50.4

SPECfp92	28.3

MIPs	100

Packaging

Single

Regulatory

FCC Class B

For more information call 1 800 INTEL4U

(Good in the US and Canada).

Intel Corporation assumes no responsibility for the use of any circuitry, other than circuitry embodied in an Intel product.  Intel486 and Pentium are trademarks of Intel.  Other brands and names are the property of their respective owners. Intel disclaims all warranties and liabilities for the use of this document  and the information contained herein, and assumes no responsibility for any errors which may appear in this document.